CENTRAL WORKSHOP

Bits-Pilani, Hyderabad Campus

Central Workshop

The Central Workshop provides training to students and caters to the maintenance & fabrication needs of the Institute. Through the course "Workshop Practice", all the first-degree students are trained to acquire the necessary skills related to various manufacturing processes like machining, casting, rapid prototyping (3D printing&3D scanning) & various trades like fitting, metrology, carpentry, smithy, foundry, sheet metal, welding, mechatronics etc. and two computer-oriented exercises, CNC Programming using CNC Simulation software (by CIMCO) and a Manufacturing Simulation software called FlexSim. Further, students of B.E. (Mechanical Engineering) are given in-depth training in various courses such as "Manufacturing Processes”, “Advanced Manufacturing Process”, "Computer-Aided Manufacturing" and for ME “Flexible Manufacturing Systems" & Machine Tool Engineering. Apart from routine maintenance and training, the workshop also provides fabrication services pertaining to the project works of students and also offers technical support for TBI works.

The workshop comprises various labs and sections namely Computer Aided Manufacturing Lab(CAM), Advanced Machining Lab, Metal Forming Lab, Powder Metallurgy Lab, Additive Manufacturing Lab, Metrology Lab, Machine shop, Welding, Fitting, Smithy, Sheet Metal, Carpentry, Foundry and Mechatronics. Major equipment include 1 Vertical Machining Center (Bridgeport VMC GX600), 1 EDM Wire cut facility (Model: SPRINT CUT), 1 CNC Lathe (Model PTC- 200), 1 Hydraulic press with computer control (40T), 9 Lathes, 2 Shapers, 1 Radial Drilling Machine, 1 Universal Milling Machine (Batlibai make) with indexing attachment, 1 Cylindrical Grinding Machine with internal grinding attachment, 1 Surface Grinding Machine, 1 Tool and Cutter Grinder, 1 Pedestal Grinder, 1 Slotting Machine, 1 Power Shearing Machine (Vivek Brand), 1 Portable Drilling Machine, 1 Injection Moulding Machine, 1 TIG Welding Machine, 1 MIG Welding Machine, 1 Spot Welding Machine, 1 Universal Milling Machine (BFW make), 1 Shaper (Sagar make), 1 Surface Roughness Tester (Mitutoyo make) and 1 Hydraulic Bulge Test Rig, 1 Electro Chemical Machining (ECM) setup, 1 Lathe Tool Dynamometer, and 1 Rotational Moulding Machine, 1 M TAB CNC Lathe Trainer, 1 3-D Coordinate Measuring Machine (CMM) “Spectra”, 1 HMT PRAGA Universal Tool & Cutter Grinding Machine, 1 FRITSCH Vibratory Sieve Shaker, Magnesium metal stir casting Furnace, 1 Milling tool dynamometer, 1 Drilling tool dynamometer, 1 Lathe tool dynamometer, 1 200 TON capacity Compression testing machine, Mold Testing Equipment, 1 Open-hearth furnace for smithy, 1 Gas-fired furnace for foundry, 5 Wood-working Lathes, 2 Arc-welding equipment’s, 1 Oxy-Acetylene gas welding equipment, 1 Gauge planner for wood works, 1 Power Hacksaw, metrology instruments,1 Sand Muller (Capacity-75kg), 5 Panther make lathe machines, 1 HMT make High Speed Precision Lathe Machine and two 3D Printers, 13 Lenovo make desktops, 15 Dell make desktops, a CNC Simulation software by CIMCO (20 Licenses),and a NC programming software called MASTER CAM (15 Licenses), 1 Plasma arc cutting machine, 3 AC Welding Transformers, 1 Powder mixing unit, 1 Metal foam fabrication set up, 1 Muffle Furnace,1 Balance(Sartorius),1 Trolley, 1 Precision Micro Machining Center,1 FRITSCH Ball Mill (Planetary Micro Mill), 1 RETSCH Pellet Press, 1 NABERTHERM High temperature Furnace,1 Voltammetry, Corrosion, and Energy System (software included) (VERSASTAT3-500),1 BFW CNC Milling, 1 Vision Measuring Machine and 1 3D Scanner with Geo magic software(Space Spider), 1 Deep Drawing Hydraulic Press(200T), 1 Three zone split furnace and ten 3d printers

In addition to the existing equipment’s, during this academic year we going to procure

1 Ultrasonic Rotary milling system and 1 Robotic welding cell.

CENTRAL WORKSHOP AREA

| LAB/TRADE | TOTAL AREA | 18500 SFT |
| --- | --- | --- |

| LAB/TRADE | m2 | ft2 |
| --- | --- | --- |
| CAM | 134.5 | 1446.5 |
| WELDING | 86.5 | 930.5 |
| FITTING & CARPENTRY | 60.2 | 647.8 |
| DIGITAL FABRICATION LAB | 85.4 | 919.7 |
| FOUNDRY/CASTING | 86.3 | 928.4 |
| MACHINE SHOP | 87.8 | 944.9 |
| SMITHY&SHEET METAL | 82.6 | 889.1 |
| LATHE | 86.6 | 931.7 |
| GRINDING | 125.0 | 1345.3 |
| METROLOGY | 35.5 | 382.6 |
| METAL FORMING | 86.5 | 930.5 |
| ADVANCED M/CING | 87.8 | 944.9 |
| MECHATRONICS &FMS | 86.6 | 932.7 |
| FOUNDRY/POWDER METALLURGY | 82.7 | 889.7 |
| WS OFFICE | 21.5 | 231.9 |

The following labs are catered by the Central Workshop

| For First Degree |  |  |  |
| --- | --- | --- | --- |
| Semester | Name of the Lab | Branch/Year | No.of Students |
| Semester 1 | Workshop Practice (ME F112) | Common for all 1 years | 550 |
|  | Manufacturing Processes Lab (ME F313) | BE. Mechanical Engineering | 200 |
| Semester 2 | Workshop Practice (ME F112) | Common for all 1 years | 550 |
|  | Advanced Manufacturing Processes Lab (MF F213) | BE. Mechanical Engineering | 200 |
|  | CAM Lab (ME F432) | BE. Mechanical Engineering | 60 |
| For Higher Degree |  |  |  |
| 1 | Flexible Manufacturing Systems(BITS F431) |  |  |
| 2. | Computer Integrated Manufacturing(ME G539) |  |  |
| 3. | Comp Aided Anal & Design (ME G611) |  |  |
| 4. | Manufacturing Planning & Control(MSE G512) |  |  |

List of Laboratories &

Major Equipment

Computer Aided Manufacturing(CAM) Lab

CNC VERTICAL MACHINING CENTER

Make			:	 BRIDGEPORT (HARDINGE)

Model			: 	GX 600

Control system	:	Fanuc 0i MD

Specifications:

| Travels: X,Y,Z |  | 600x540x540mm |
| --- | --- | --- |
| Working Surface |  | 750mm x 540mm |
| Control Type |  | Fanuc i-Series GX |
| Spindle Taper |  | No. 40 |
| Spindle power |  | 15kw |
| Spindle Speed |  | 8,000 rpm |
| Tool Changer capacity |  | 20 |
| Accuracy | Positioning | 0.01mm |
|  | Repeatability | 0.005mm |
| Feed rates max. | Cutting | 12m/min |
|  | Rapid &Jog | 30m/min |

CNC VERTICAL MACHINING CENTRE

Make			: BFW

Model                     : CHANDRA+

Control System       : Fanuc OiMF +

Specifications:

| Travel: X,Y,Z |  | 800 x 350 x 380 mm |
| --- | --- | --- |
| Working Surface |  | 1060 x 315 mm |
| Spindle Power |  | 5.5 kW |
| Spindle Speed |  | 60 - 6000 rpm |
| Tool Changer Capacity |  | 20 |
| Accuracy | Positioning | ± 0.010 mm |
|  | Repeatability | ± 0.005 mm |
| Feed Rates Max. | Cutting | 1-5000 mm/min. |
|  | Rapid& Jog (X,Y) | 10,000 mm/min. |
|  | Rapid & Jog (Z) | 5000 mm/min. |

CNC LATHE MACHINE

Make				:	HMT-PRAGA

Model				:	PTC- 200

Control system		:	FANUC Oi-Mate

SPECIFICATIONS:

| Swing diameter over Bed |  | ø250mm |
| --- | --- | --- |
| Max.Turning Diameter |  | ø200mm |
| Max.Turning Length with Chuck |  | 200mm |
| Max.Turning Length without Chuck |  | 300mm |
| Spindle Speed |  | 100 – 4000rpm |
| No. of Tools |  | 8 |
| Turning Tool Size |  | 20x20mm |
| Type of Bed |  | Inclined 30˚ to vertical |
| Traverse | X-axis | 110mm |
|  | Z-axis | 300mm |
| Feed Rates |  | 1 – 5000mm/min |
| Rapid Feed Rates | X-Axis | 18m/min |
|  | Z-axis | 30m/min |
| Positional | X-Axis | ±0.003mm |
|  | Z-axis | ±0.004mm |
| Repeatability | X-Axis | ±0.003mm |
|  | Z-axis | ±0.004mm |

CNC TURNING CENTER –TRAINER

Make: MTAB Engineers Pvt. Ltd

Model: XLTURN

# Specifications:

| X- Axis Travel | 80 mm |
| --- | --- |
| Z- Axis Travel | 180 mm |
| Chuck Size | 100 mm |
| Maximum Turning Diameter | 32 mm |
| Maximum Turning Length | 120 mm |
| Distance between Centers from Spindle Face | 210 mm |
| Spindle Nose Taper | A2-3 / MT3 |
| Spindle Speed Range | 150 - 3000 RPM |
| Spindle Motor Capacity | 1 HP |
| Number of Stations | 8 |
| Control | Fanuc |
| Main Supply | Single phase, 110V/230 V, 50/60 Hz |

HIGH SPEED MICROMACHINING CENTER

Make: MicroMach, India

Model: PMC080

Specifications:

| Stage Travel | X-axis | 100 mm |
| --- | --- | --- |
|  | Y-axis | 100 mm |
|  | Z-axis | 100 mm |
| Mode of Operation |  | CNC |
| Accuracy |  | ± 2µm for stage |
| Resolution |  | 0.15 µm |
| Load Capacity |  | 15 kg on stage |
| Max. Spindle Speed |  | 80,000 RPM |
| Travel Speed |  | 0.01 – 20 mm/sec |
| Max. Torque |  | 11.9 N-cm |
| Support |  | Granite |

LATHE CUTTING FORCE DYNAMOMETER

Make: NK Instruments

Specifications:

| 3 Component Piezoelectric Based Cutting Force Measurement System( Fz, Fx, & Fy) |  |
| --- | --- |
| Cutting force Dynamometer Size | 109 x 80x 35 mm |
| Range: Fx , Fy & Fz direction | ± 4 kN, |
| Sensitivity | Fx: 3.7 pC/N, Fy: 7.8 pC/N, Fz – 7.8 pC/N. |
| Cable: | Connecting cable for 3-component sensors with V3 neg. connector - 3xBNC pos- 3 to 5 meter length |
| Multi-Channel Charge Amplifier | Industrial type Charge Amplifier, 3 channel , BNC, Range 15PC to 1,00,000 PC, Voltage Output:+/-10 V, Max Current Output+/- 5 mA, Output Impedance: 10 ohm |
| Data Acquisition System | Data Acquisition Hardware supporting 4 Channel Simultaneous Sampling, Input Range: ±1.25,±2.5,±5,±10 V, Sample Rate: 50.0/100 kS/s per Channel, Input BNC neg. Resolution: 24-Bit Anti-aliasing Filter, Interface: USB 2.0 |
| Data Acquisition Software Data Acquisition & Analysis Software | Customized Standalone configuration based on Channel selection. Graphics Functions Measured data represented on one or more graphs Display as y(t), Display of numeric values Cursor function, Zoom function, Data Export with storing facility. Display of average value of data Min, Max value of data. |

CNC MILLING, LATHE & DRILLING TOOL DYNAMOMETERS

Make: EMI Technologies

Specifications:

| Drilling Tool Dynamometer (2 Forces – Thrust & Torque) With Signal conditioner for XY axis. |  |
| --- | --- |
| Sensor- | Strain gauge based two axis force sensor |
| Accuracy(Approx.) | ± 1 of full scale |
| Linearity(Approx.) | ± 1 of full scale |
| Excitation Voltage (Approx.) | 10V DC MAX |
| Display Type | Independent for each axis. |
| Range | 500 kg force in Thrust 20.0kgm force in Torque |
| USB data acquisition system (2 Channel) | Resolution: 12-bit ADC, No of samples: 10K samples per sec. No of channels:8(single ended),4(differential ended) |
| Lathe Tool Dynamometer with Signal Conditioner for XYZ |  |
| Accuracy(Approx.) | ± 1 of full scale |
| Linearity(Approx.) | ± 1 of full scale |
| Excitation Voltage (Approx.) | 10V DC MAX |
| Display Type | Independent for each axis. |
| Range | 500 kg force in three direction |
| USB data acquisition system (3 Channel) | Resolution:12 bit ADC, No of samples: 10K samples per sec. No of channels:8(single ended),4(differential ended) |
| Milling Tool Dynamometer With Signal conditioner for XYZ axis. |  |
| Sensor Size(Top) | 300mm X 300mm |
| Sensor | Stain gauge based three axis force sensor |
| Accuracy(Approx | ± 1 of full scale |
| Linearity(Approx) | :-:- ± 1 of full scale |
| Excitation Voltage (Approx) | 10V DC MAX |
| Display Type | Independent for each axis |
| Range | 500 kg force in three direction |
| USB data acquisition system (3 Channel) | Resolution:12 bit ADC, No of samples: 10K samples per sec. No of channels:8(single ended),4(differential ended) |

TOOL VIBRATION MEASUREMENT DEVICE

Make: PCB Piezotronics

Model: 356B21&482C05

# Specifications:

| Platinum Stock Products: Triaxial ICP® ACCELEROMETER |  |
| --- | --- |
| Sensitivity | (±10%)10 mV/g (1.02 mV/(m/s²)) |
| Measurement Range | ±500 g pk (±4905 m/s² pk) |
| Electrical Connector | 8-36 4-Pin |
| Frequency Range (±5 %) (y or z axis) | 2 to 10000 Hz |
| Frequency Range (±5 %) (x axis) | 2 to 70 00 Hz |
| Resonant Frequency | ≥55 kHz |
| Broadband Resolution (1) | 0.004 g rms (0.04 m/s² rms) |
| Platinum Stock Products; 4-channel, line-powered, ICP® sensor signal conditioner |  |
| Channels | 4 |
| Sensor Input Type(s) | ICP® |
| Output Range (Maximum) | ±10 V |
| Low Frequency Response (-5 %) | <0.1 Hz |
| High Frequency Response (-5 %) | >1000 kHz |
| Phase Response (at 1 kHz) | ±1 ° |
| Cross Talk (maximum) | -72 dB |
| Fault/Bias Monitor/Meter (LED) | Open/Short/Overload |

Advanced Machining Lab

CNC WIRECUT EDM

Make				:	Electronica

Model				:	Sprintcut

Specifications:

| Main table traverse (X,Y) | 300 mm x 400 mm |
| --- | --- |
| Aux. table traverse (u,v) | 80 x 80 mm |
| Max. work piece height | 200 mm |
| Taper | ± 30° over 50 mm |
| Max. JOG speed | 900 mm/min |
| Best surface finish | 0.8 µm |
| Wire electrode diameter | 0.25(std.), 0.15, 0.2 mm (opt.) |
| Resolution | 0.0005 mm |
| Least input increment | 0.001 mm |
| Capacity | 10 kVA |

ELECTRO CHEMICAL MACHINING SYSTEM

(Table Top Model)

Make		: METATECH Industries

Model 	: ELECTRO-M

Specifications

| Tool Area | 300 mm2 (10x30mm or 15x20mm) |
| --- | --- |
| Cross Head Stroke | 40mm |
| Supply | Single phase 230V. A.C |
| Electrical Output Rating | 0-100Amps and Voltage 0-25V DC |
| Tool Feed Rate | 0.2 to 1 mm/min |
| Machining Time | 0 to 1999seconds |

Metal Forming Lab

HYDRAULIC PRESS (Deep Drawing Test Setup)

Make: J. RAGRAU INSTRUMENTS

Specifications:

| Capacity | 40T |
| --- | --- |
| Usable range | 35T |
| Displacement | 500 mm |
| Stroke length | 400 mm |
| Holding pressure | Up to 50 bar |
| Working pressure | Up to 100 bar |
| Daylight | 600 mm |
| Working area | 600 x 600 mm |
| Power pack motor | 7.5 h.p |

HYDRAULIC COMPRESSION TESTING MACHINE

Make		: HEICO

Model 	: 2000kN

Specifications:

| Capacity | 2000 kN |
| --- | --- |
| Vertical Clearance | 800mm |
| Travel of the Ram | 100mm |
| Platen Size | 250 mm square |
| HYDRAULIC POWER PACK |  |
| Flow of the pump | 10LPM |
| Motor capacity | 5 H.P. |
| Capacity of the tank | 100litres |
| Operating pressure | 200 Bars |
| Noise Level | Less than 70dB |

DIGITAL HAND OPERATED BULGE TESTING SYSTEM

Make	: J. RAGRAU INSTRUMENTS

SHEARING MACHINE

MAKE :  VIVEK

MODEL :  VUS 7

Specifications :

| Cutting Capacity (MS) Length x Thickness | 1525 X 4 mm |
| --- | --- |
| Stroke per minute | 50 |
| Table height | 800 mm |
| Table width | 400 mm |
| Front gauge | 600 mm |
| Rear gauge | 750 mm |
| Blade length x width x thickness | 1525x63x18 mm |
| Motor capacity | 3.7kW |

MECHANICAL PRESS

MAKE :  VIVEK

MODEL :  VPP – 3 (C-Type)

Specifications :

| Capacity | 30 Ton |
| --- | --- |
| Stroke adjustment | 63 mm |
| Length and width bed | 522X381 mm |
| Distance bed to ram | 254 mm |
| Stroke per min. (s.g.) | 45 |
| Stroke per min. (d.g.) | 40 |
| Floor to top of bed | 890 mm |
| Motor capacity | 2.2 kW |

Metrology Lab

COORDINATE MEASURING MACHINE(CMM)

Make	:	Accurate India.

Model	:	Spectra

Specifications:

| Measuring range( mm) | X axis | 500mm |
| --- | --- | --- |
|  | Y axis | 600mm |
|  | Z axis | 400mm |
| Mode of operation |  | Motorised / CNC |
| Accuracy MPEE Maximum Permissible Error For Length Measurement (As per ISO 10360-2 with TP 200 in µm) |  | 2.2+L / 350 |
| Resolution (in µm) |  | 0.1 / 0.5 |
| Max 3D Velocity (mm/sec) |  | 900 |
| Acceleration (mm/sec2) |  | 1200 |
| Max. Workpiece weight (kg) |  | 250 |
| System Weight (kg) |  | 890 |
| Guidance |  | Air Bearings on all axes |
| Measuring table (Material) |  | Granite |
| Probing Option |  | TP 20, TP200, SP 25M, Revo, Apollo, laser scanner |
| Probe heads |  | MH8, MIH, PH 10, PH 20 |

SURFACE ROUGHNESS TESTER

Make		:	Mitutoyo

Model		:	SJ 410

Specifications:

| Differential Induction Method |  |
| --- | --- |
| Measuring range | 25 mm |
| Resolution | 0.000125 µm |
| Stylus | Dimond |
| Tip radius | 5 µm |
| Measuring force | 4 mN |

3D SCANNER

Make		:	Artec3D

Model		:	Artec Space Spider

Specifications:

| 3D resolution, up to | 0.1 mm |
| --- | --- |
| 3D point accuracy, up to | 0.05 mm |
| 3D accuracy over dist. up to | 0.03% over 100 cm |
| Texture resolution | 1.3 MP |
| Colors | 24 bpp |
| 3D Light source | blue LED |
| 2D Light source | 6 white LED |
| Working distance | 0.2 – 0.3 m |
| Linear field of view, HxW @closest range | 90 mm x 70 mm |
| Linear field of view, HxW @furthest range | 180 mm x 140 mm |
| Angular field of view, HхW | 30 x 21° |
| Video frame rate, up to | 7.5 fps |
| Data acquisition speed, up to | 1 000 000 points/s |
| Multi core processing | Yes |
| Weight | 0.8 kg / 1.8 lb |
| Interface | 1 х USB3.0 |
| Output formats | OBJ, PLY, WRL, STL, AOP,ASCII, PTX, E57, XYZRGB |
| Output format for measurements | CSV, DXF, XML |
| Reverse Engineering Software | Geomagic Design X |

VISION MEASURING MACHINE

Make		:	Mitutoyo

Model		:	Quick Scope--L2010Z/AFC

Specifications:

| XYZ Measuring volume |  | 200 x 100 x 150 mm |
| --- | --- | --- |
| Resolution/Scale type |  | 0.1 μm / Linear encoder |
| Drive method |  | X / Y axis: manual, Z axis: CNC with Auto focus |
| Accuracy | X-axis | (2.2+0.02L) µm, L in mm |
|  | Y-axis | (4.5+0.006L) µm, L in mm |
| Image Sensor |  | 3 mega pixel CMOS colour Camera |
| Observation unit |  | 8-step zoom lens; total magnification: 29X to 202X |
| Illumination | Transmitted light | White LED |
|  | Co-axial light | White LED |
|  | Ring light | 4 – quadrant ring light(LED) |
| Maximum stage loading |  | 10 kg |

Additive Manufacturing Lab

3D PRINTER

Make		:	RAISE3D

## Model		:	N2 Dual 3D Printer

Specifications:

| Technology | FDM |
| --- | --- |
| Build Volume | 305 x 305 x 610 mm |
| Layer Resolution | 10 – 300 Microns |
| Extruders | Dual |
| Filament Diameter | 1.75 mm |
| Supported Filament | PLA / ABS |
| Nozzle Diameter | 0.4 mm |

3D PRINTER

Make		:	FLASHFORGE

## Model		:	Finder

Specifications:

| Technology | FDM |
| --- | --- |
| Build Volume | 140 X 140 x 140 mm |
| Layer Resolution | 100 – 500 Microns |
| Extruders | Single |
| Filament Diameter | 1.75 mm |
| Supported Filament | PLA / PVA |
| Nozzle Diameter | 0.4 mm |

Foundry Lab

# MAGNESIUM METAL STIR CASTING FURNACE

# Make: VB Ceramics

# Model: VBCC / MM-SCF / 1000°C

### Specifications:

| CRUCIBEL VOLUME | 200mm Di. X 300mm |
| --- | --- |
| Operating TEMPERATURE | 1000°C (Max. 1100°C) |
| THERMOCOUPLE | K – Type (225mm long) with 1°C accuracy |
| POWER | 4KW – SINGLE PHASE/AC/230V |
| Furnace Actual Current Rating | 15 Amps (Max) |
| Stirrer Motor Capacity and Speed | AC motor with 0.25HP and 1400rpm |
| Die material | Cast Iron |

COMPOSITE MATERIALS FABRICATION EQUIPMENT

Make: VB Ceramics

Specifications:

| Mould Volume | Dia. 50 mm X height 150 mm |
| --- | --- |
| Operating Temperature | 1000oC (Max 1100oC) |
| Thermocouple | K Type (225 mm Long) with 1oC accuracy |
| Power | 4kW- Two Phase/ AC/ 230 V |
| Furnace Actual Current Rating | 15 Amps (Max.) |
| Working Atmosphere | Vacuum/ Inert Gas (Up to 5 Bar) |
| Die Material | Stainless Steel |

INJECTION MOULDING MACHINE

Make:  TEXSHINE

Model:  7 HL

Specifications:

| ###### Maximum mold size | ###### 250 x 250 mm |
| --- | --- |
| ###### Maximum short weight | ###### 60 grams |
| ###### Maximum daylight opening | ###### 450 mm |
| ###### Motor power | ###### 3.75 kW |
| ###### Heater load | ###### 2.5 kW |
| ###### Machine operating pressure | ###### 105 bar |
| ###### Electrical system | ###### Time based control |
| ###### Maximum temperature | ###### 220C |

ROTATIONAL MOULDING MACHINE

Make		: VINODRAI Engineers Pvt. Ltd

Model	: EN 200 PLC

Specifications

| Suitable For | Making plastic water storage tank of 200 liters or plastic articles |
| --- | --- |
| Power Requirement | 3HP 415V 3Phase |
| Control | PLC with Touch Screen |
| Fuel | LPG |
| Material Used | LLDPE |
| Rotation | Biaxial |
| Major(X) & Minor(Y) Axis Speed Range in RPM | 0–8 |
| Maximum Temperature | 210C |
| Sensing Device | Wireless temperature sensing device for measuring peak internal air temperature of the mould |
| Size Of The Machine (LxWxH) in m | 3.4 x 2.4 x 2.0 |

SAND MULLER

Supplier: METSONIC ENGINEERS PVT LTD

Specifications

| Capacity | 75 kg. |
| --- | --- |
| Pan diameter (internal) X ht | 900 x 400 mm |
| Roller diameter X width | 400 x 90 mm |
| Motor | 3 hp 1440 rpm |
| Rotation | clockwise |
| Spindle rpm | 32 |
| Mulling time | 4 to 5 minutes |

Welding Lab

PLASMA PROFILE CUTTING MACHINE

Make: ADOR

Model: Kingcut Smarty

Specifications:

| Machine overall width (without Cable Track) | 1600 mm Aprox |
| --- | --- |
| Rail length: | 3500 mm Aprox |
| Cutting width | 1500 mm |
| Cutting length | 3000 mm |
| Material to cut with OXY Fuel process: | Mild steel |
| Machine rapid speed: | 6000 mm/Min |
| Gas Required | Oxygen and Acetelene or LPG |
| Cutting Thickness by Plasma | Max 16 mm with Pierce 20 mm from Edge |
| Cutting Thickness by Gas | 5-150 mm |
| Programing accuracy | ± 0.1 mm |
| Repeatability Accuracy | ± 0.5 mm |

TIG WELDING MACHINE

(Inverter based, digitally controlled, AC/DC pulse TIG-MMA Welding Machine (gas cooled))

Model: V 24 AC/DC, Mobile, Gas Cooled

Make: LORCH Schweisstechnik GmbH, Germany

Welding Range: 3 - 240 A

MIG WELDING MACHINE

(Digital microprocessor controlled, Synergic Pulse MIG-MAG welding)

Make: LORCH Schweisstechnik GmbH, Germany

Model: S3 pulse Mobile Standard, Gas-cooled

Welding Range: 25-320 A

Synergic program for: Steel, Stainless Steel, Aluminum & Copper

SPOT WELDING MACHINE

MAKE: GEMCO

Model:

Specifications:

| Rating | 4 KVA |
| --- | --- |
| Main Supply | 230 V/50 Hz. Single Phase |
| Throat Depth | 125mm |
| - Weld ability in M.M.: M.S., S.S., Brass. |  |
| - Maximum Thickness x 2: 0.8, 0.5, 0. |  |

WELDING GENERATOR

Make: ADOR

Model: SUPERGEN 320

Specifications:

| Supply voltage | 415 Volts |
| --- | --- |
| Phase | 3 |
| Power | 15.5/20.78 Kw/HF |
| Open circuit voltage | 100 Volts |
| Welding current range | 35 – 320 Amps |
| Type of cooling | Forced Air |

AC ARC WELDING TRANSFORMER

Make: ADOR

Model: TPA 303

Specifications:

| Supply voltage | 380/415 Volts |
| --- | --- |
| Phase | 3 |
| Power | 16.5 KVA |
| Open circuit voltage | 66 Volts |
| Welding current range | 40-300 Amps |
| Type of cooling | Forced Air |

Powder Metallurgy Lab

PLANETARY MICRO MILL

## Model: PULVERISETTE 7(premium line)

Make: FRITSCH GmbH.

Specifications:

| Application | Synthesis of micro, nano particles, mechanical alloying |
| --- | --- |
| Working Principle | Impact force |
| Grinding stations | 2 No’s |
| Grinding Bowls-Volume | 80 mL, 20 mL |
| Bowl and Balls-material | Tungsten Carbide |
| Grinding ball diameter | 0.6, 5, 10 mm |
| Max. feed size | 5 mm |
| Grinding conditions | Dry/wet/inert |
| RPM | Max. – 1100 Operation - 1000 |
| Attachments | GTM, Wet process emptying device, counterweights, inert gas |

PELLET PRESS

Model: PP 25

Make: RETSCH

Specifications:

| Application | Pressing pellets from powders |
| --- | --- |
| Feed Grain Size | <60 µm or <40 µm |
| Max. Capacity | 25 Ton |
| Operating capacity | 20 Ton |
| Pressure force | 10-250 kN |
| Die diameter | 32 mm |
| Attachment | Aluminum cups, free pressing |

HIGH TEMPERATURE MUFFLE FURNACE

Make: NABERTHERM

Model: LHTC(T) 03/16

Specifications:

| Application | Sintering of metals and ceramics |
| --- | --- |
| Max. temperature | 1600 °C |
| Working temperature | 1500 °C |
| Heating chamber dimensions | 120x210x120 mm |
| Heating elements | SiC rods |
| Features | Programable, retrieval of data, fresh air flap, exhaust pipe |
| Attachments | Rotameter, inert atmosphere |

POTENTIOSTAT AND GALVANOSTAT

Make: Princeton Applied research

Model: VersaSTAT 3

Specifications:

| Application | Energy Storage, Corrosion & Electrochemistry. |
| --- | --- |
| Voltage control (Potentiostat mode) |  |
| Applied voltage range | ± 10 V |
| Applied voltage accuracy | ±0.2% of value ± 2 mV |
| Current control (Galvanostat mode) |  |
| Applied current range | ± 2 A |
| Minimum current range / resolution | ± 200 nA / 60 pA |
| Impedance (EIS) |  |
| Mode | Potentiostatic / Galvanostatic |
| Frequency range | 1 MHz |
| Software | VersaStudio |
| Attachments | Corrosion cell, platinum, calomel, Ag/AgCl reference electrodes. |

WEIGHING BALANCE

Model	: Quintix 6102 -10IN

Make	: Sartorius

Specifications:

| Weighing Capacity | 6100 g |
| --- | --- |
| Readability | 10 mg |
| Repeatability | 10 mg |
| Linearity Deviation | 20 mg |
| Weighing Pan Size | Ø180 mm |

MUFFLE FURNACE

(With PID Controller & Digital Timer)

Model	: MSI – 73

Make	: META Lab

Specifications:

| Temp. Range | 1200oc |
| --- | --- |
| Working Temp. | 1150oc |
| Accuracy | +/- 1% FSD |
| Power Rating | 3.5 K.W |
| Size | 12"X6"X6" |

## VIBRATORY SIEVE SHAKER

## Make	: M/s. FRITSCH GmbH

## Model: ANALYSETTE 3 PRO

Specifications:

| Method of analysis | sieving |
| --- | --- |
| Sieving action | two-dimensional |
| Dry sieving |  |
| Measuring range | 20 µm - 63 mm* |
| Max. sample quantity approx | for sieves < 63mm:upto 2 kg* for sieves < 100 µm: up to 100 g* |
| Sieving time approx. | 3 - 20 min* |
| Wet sieving |  |
| Measuring range | 20 µm - 10 mm |
| Max. sample quantity approx. | 20 - 100 g* |
| Sieving time approx. | 30 - 60 min* |
| Further details |  |
| Sieve diameters | 100 mm, 200 mm or 8" |
| Max. number of sieves per sieve stack | 10 (50 mm height), 16 (25 mm height) |
| Max. weight of sieve stack | 3 kg |
| Max. height of sieve stack | 550 mm |
| Amplitude | 0.1 - 3 mm |

Machining Shop

UNIVERSAL MILLING MACHINE

Model: UF 2

Make: BFW

###### Specifications:

| ###### Longitudinal movement |  |
| --- | --- |
| ###### Automatic | ###### 800 mm |
| ###### Manual | ###### 830 mm |
| ###### Transverse movement |  |
| ###### Automatic | ###### 230 mm |
| ###### Manual | ###### 260 mm |
| ###### Spindle taper | ###### ISO 40 |
| ###### Speed range | ###### 45 – 2000 rpm |
| ###### Clamping area | ###### 1350 mm x 315 mm |
| ###### T-slot / width / CD | ###### 3 Nos/14 mm/63 mm |
| ###### Feeds: |  |
| ###### Longitudinal | ###### 16 – 800 mm/min |
| ###### Cross | ###### 16 – 800 mm/min |
| ###### Vertical | ###### 4 – 200 mm/min |
| ###### Swivel range of table to both sides | ###### 45 |
| ###### Milling spindle Motor | ###### 5.5 kW |
| ###### Feed Motor | ###### 1.5 kW |
| ###### Maximum safe load on table | ###### 400 kg |

HORIZONTAL / VERTICAL MILLING MACHINE

Make  : BATLIBOI

Model : BFH-5

Specifications  :

| ###### Longitudinal movement |  |
| --- | --- |
| ###### Automatic | ###### 810 mm |
| ###### Manual | ###### 810 mm |
| ###### Transverse movement |  |
| ###### Automatic | ###### 275 mm |
| ###### Manual | ###### 280 mm |
| ###### Spindle taper | ###### ISO 40 |
| ###### Speed range | ###### 45 – 2000 rpm |
| ###### Clamping area | ###### 1350 mm x 320 mm |
| ###### T-slot / width / CD | ###### 3 Nos /14 mm / 63 mm |
| ###### Feeds: |  |
| ###### Longitudinal | ###### 14 – 900 mm/min |
| ###### Vertical | ###### 4 – 250 mm/min |
| ###### Cross | ###### 14 – 900 mm/min |
| ###### Milling spindle Motor | ###### 5.5 kW |
| ###### Feed Motor | ###### 1.5 kW |
| ###### Maximum safe load on table | ###### 275 kg |

SHAPING MACHINE

Make :  SAGAR

Model :  SM 18 “

Specifications :

| ###### Stroke length | ###### 18 “ |
| --- | --- |
| ###### Working surface of table | ###### 407mm X 292mm |
| ###### Maximum size of tool shank | ###### 32mmX16mm |
| ###### Range of ram speeds (Stroke Per Minute) | ###### 17,25,50,75 |
| ###### Motor Capacity & Speed | ###### 1.5KW &960rpm |

VERTICAL SLOTTING MACHINE

Make  :  Daljith

Model  :  DS –10

Specifications  :

| ###### Stroke | ###### 10’’ |
| --- | --- |
| ###### Rotary table | ###### 15’’ |
| ###### Longitudinal movement | ###### 12’’ |
| ###### Cross movement | ###### 10’’ |
| ###### Cutting Speed of RAM (Per Minute) | ###### 40x60 |
| ###### RAM Adjustment | ###### 12’’ |
| ###### Throat Depth | ###### 11’’ |
| ###### Distance from the Tool to the Main Column | ###### 14’’ |
| ###### Motor Capacity | ###### 1.5kw |

# HIGH SPEED PRECISION LATHE MACHINE

## Make		: 	HMT Ltd

# Model		: 	NH26/1500

# Specifications:

| Height of centres | 260 mm |
| --- | --- |
| Dist. between centres | 1500 mm |
| Swing over bed | 575 mm |
| Swing over cross slide | 350 mm |
| Spindle Nose/Bore | A2-6/53 mm |
| Feed range (longitudinal) | 0.04 – 2.4mm/rev |
| Feed range (cross) | 0.02 – 1.2mm/rev |
| Spindle speed range | 40 – 2040rpm |
| Spindle power | 11KW |

RADIAL DRILLING MACHINE

# Make: Batlibai

# Model: BVR 3

# Specifications:

| Max.drilling diameter in Steel | 32 mm |
| --- | --- |
| Max.drilling diameter CI | 35 mm |
| Drilling Radius max / min | 930 / 340 mm |
| Vertical Traverse of Arm | 520 mm |
| Horizontal Movement of Drill Head | 590 mm |
| Drilling Motor Power | 1.5 kw |

# LATHE MACHINE (7NOS)

## Make: Gujarat Lathe Manufacturing Co. Pvt. Ltd

# Model: PANTHER 1350 / 1 Series

# Specifications:

| Swing | 335 mm |
| --- | --- |
| Dist. between centres | 540 mm |
| Swing over slide | 200 mm |
| Slide movement | 210 mm |
| Spindle Bore | 42 mm |
| Taper bore in center sleeve | MT 4 |
| No. of speeds Range / RPM | 8 45-938 |
| Motor capacity | 1.5 kW |

# LATHE MACHINE(2NOS)

## Make: Gujarat Lathe Manufacturing Co. Pvt. Ltd

# Model: PANTHER 1650 / 1 Series

# Specifications:

| Swing | 410 mm |
| --- | --- |
| Dist. Between Centres | 1000 mm |
| Swing over Slide | 250 mm |
| Slide Movement | 250 mm |
| Spindle Bore | 42 mm |
| Taper Bore in Center Sleeve | MT 4 |
| No. of Speeds Range / RPM | 8 45-938 |
| Motor capacity | 2.2 kW |

# SURFACE GRINDING MACHINE

Make: HMT Machine Tools Limited

# Model: 452 P

Specifications :

| Work Table | 625 x 220 mm |
| --- | --- |
| Spindle Motor | 1.5 kW |
| Vertical Traverse | 280 mm |
| Grinding Wheel Size | 200x20x31.75 mm |
| Spindle Speed | 2800 rpm |

UNIVERSAL TOOL & CUTTER GRINDING MACHINE

Make: HMT Machine Tools Limited

# Model: 411

# Specifications:

| Work Area | 750X 110 mm |
| --- | --- |
| Max. Swing on Cutter Head | 305 mm |
| Circular adjustment | 360 |
| Wheel Head Motor | 0.75 kW |
| Swivel in Vertical plane | 180 |
| Vertical Adjustment | 200 mm |

# UNIVERSAL CYLINDRICAL GRINDING MACHINE

# Make: HI-LIFE MACHINE TOOLS LIMITED

# Model: HL 500

### Specifications:

| Centre Height | 150 mm |
| --- | --- |
| Swing Diameter | 295 mm |
| Max. Grinding Length | 500 mm |
| Grinding Wheel Dia. | 400 mm |
| Grinding wheel width | 40 mm |
| Wheel Head Swivel | + / - 60 |
| Work head Centre | MT 5 |
| Tailstock Centre | MT 4 |
| Internal Grinding dia. range | 15-100 mm |

# PEDESTAL GRINDER

### Make		: EIFCO Machine Tools (P) Ltd

# Model		: MG 12

### Specifications:

| Size of grinding wheel | 300 x 40 x 50.8 mm |
| --- | --- |
| Wheel Centre distance | 645 mm |
| Base to centre line of spindle | 860 mm |
| Diameter of spindle bearings | 35 mm |
| Power of motor | 2 HP |
| Speed of Motor (rpm) | 1400 rpm |
| RPM of Grinding Wheels | 1910 rpm |

### POWER HACKSAW

### Make: EIFCO Machine Tools (P) Ltd

### Model: HP 300

### Specifications:

| Max. Diameter | 300 mm |
| --- | --- |
| Max. Square | 250 mm |
| Stroke length (non adjustable) | 160 mm |
| No. of speed strokes | 3 |
| Range of speed strokes (per minute) | 80 - 100 - 125 |
| Blade Size | 525 X 45 X 2.25 mm |
| Power of main motor | 1.5 kW |
| Power of coolant motor | 0.09 kW |

Carpentry Section

# MULTIFUNCTION WOOD WORKING MACHINE

# MAKE: JAI

# MODEL: J-1010

### Specifications:

| Surfacing capacity | 10" |
| --- | --- |
| Max.planing width | 10" |
| Thicknessing capacity | 7" |
| Feeding speed-feet per minute | 21 |
| Cutter block speed- RPM | 4000 |
| Circular saw table size | 10" |
| Max.circular saw diameter | 12" |
| Power required - HP / RPM | 3/1440 |

# WOOD LATHE (5 NOS)

# Make: JAI

# Model: J 911

### Specifications:

| Length of bed | 54" |
| --- | --- |
| Width of bed | 8 ½" |
| Height of centers | 6½" |
| Admit between centers | 30" |
| Approx. spindle speed - RPM | 700 / 1400 / 2800 |
| Face plate diameter | 7½" |
| Power required HP/RPM | 1 / 1440 |